## IV – LASER GROUP 2

## Sta. Maria, Florian Antoinette           Maata, Myles Dominique	        Bungco, Vanessa Ivy

## Garingo, Juan Khaleel			Boniao, Sherhanna Maureen

## CANDLE – MAKING PROCESS

## Materials/Chemicals:

- wax

- wicks

- mold or containers

- wax melter

- candle making thermometer

- fragrances

- dyes

- putty for molds

PROCEDURE:

## The Mixing Process

- To begin, spread newspapers around the candle making area

- Melt your paraffin or soy wax and it must be double-boiled. Usually you place a large pot that is about half-filled with water on a burner over low-medium heat, place a melter in the water, then gradually place wax pieces into the melter.

- When the wax has melted, you can add coloring or fragrance as desired. The mixing process begins with the melting of the wax. This is done at 130 degrees F.

- While the wax is melting, the wick is measured at 2 inches than the candle, and then is placed in the center of the mold.

- The stearic acid and dye are then mixed and dissolved in a pot.

- Once the mixture reaches 190 degrees F, all ingredients are added, stirred and blended. The temperatures during this process are maintained at the same 190 degrees.

## Molding Process

- After all the ingredients have been blended, the wax is then poured into the mold. The mold forms the actual shape of the candle. The wax is allowed to cool.

- During the cooling process, the wax contracts and creates a small well around the wick. When the wax is cooled thoroughly, the candle is removed.

- The final product should have an even distribution of color, a smooth surface and a pleasant but not overwhelming fragrance.

Source: http://www.ehow.com/how-does_5117892_process-candle-making.html#ixzz0tYWbrhLe