How to Make Lye Soap With Lard

Members:

Madcasim, Nasser

Ruelo, Earl Vincent

Cañaveral, Cherlene Hope

Poss, Maria Paula

Objectives:

In this lab experiment, we aim to be able to:

- Make soap out of commonly available materials.

- Learn the process of soap making properly.

- Learn the applications of soap making.

Things You'll Need:

12 oz lye

21 1/2 oz cold water

5 lbs 7 1/3 oz lard Equipment

Plastic dish pan

Non-reactive pot to melt lard

wooden spoon

soap molds or wooden box

thermometer Safety Equipment

rubber gloves

goggles

10% solution vinegar and water

Instructions:

- Put on gloves and goggles. Normally, safety advice would be in the warnings section, but this is a vital step when you are working with lye. Lye is extremely caustic. It will burn your skin and can damage your eyes permanently. Wear gloves and goggles at all times when working with lye.

- Pour ice cold water into plastic dish pan. Work in a well-ventilated area to avoid breathing in fumes from the lye.

- Slowly add lye to the cold water, stirring constantly and slowly with the wooden spoon. If any of the lye splatters on you, wash it off immediately with lots of cold water. The lye will react chemically with the water and begin to heat it. Cover and set aside till it reaches 85 F.

- Melt lard in non-reactive pan or bowl. Cool lard to about 95 F.

- Carefully pour the lye water into the lard in a thin, continuous stream, stirring constantly with wooden spoon in a slow S-motion. Continue to stir until soap thickens to the consistency of pudding. The soap is ready to pour when a drop of soap dropped onto the surface of the soap does not sink into the rest of the mixture. This can take up to 45 minutes.

- Pour soap mixture into prepared molds. Treat the soap mixture just as carefully as you did the lye water, and clean up any drips with a sponge soaked in 10% vinegar solution.

- After the soap has cured for 3-5 hours, you can use a butter knife to cut it into bars or slices. Do not remove it from the box or the molds.

- Let the soap cure in the box for about a week before removing it. Wrap the bars in muslin and store in a cool place to cure for another 3-4 weeks before using it.

Source: How to Make Lye Soap With Lard | eHow.com http://www.ehow.com/how_4695940_lye-soap-lard.html#ixzz104JXAx21